Porovnanie médií používaných v telekomunikačných sieťach

| Media Type | Maximum Segment Length | Speed | Cost | Advantages | Disadvantages |
| --- | --- | --- | --- | --- | --- |
| UTP | 100 m | 10 Mbps to 1000 Mbps | Least expensive | Easy to install; widely available and widely used | Susceptible to interference; can cover only a limited distance |
| STP | 100 m | 10 Mbps to 100 Mbps | More expensive than UTP | Reduced crosstalk; more resistant to EMI than Thinnet or UTP | Difficult to work with; can cover only a limited distance |
| Coaxial | 500 m (Thicknet) 185 m (Thinnet) | 10 Mbps to 100 Mbps | Relatively inexpensive, but more costly than UTP | Less susceptible to EMI interference than other types of copper media | Difficult to work with (Thicknet); limited bandwidth; limited application (Thinnet); damage to cable can bring down entire network |
| Fiber-Optic | 10 km and farther (single-mode) 2 km and farther (multimode) | 100 Mbps to 100 Gbps (single mode) 100 Mbps to 9.92 Gbps (multimode) | Expensive | Cannot be tapped, so security is better; can be used over great distances; is not susceptible to EMI; has a higher data rate than coaxial and twisted-pair cable | Difficult to terminate |

[Cisco]

There are four most common Unshielded Twisted Pair Ethernet standards available nowdays (info z 13. sep.2015, http://pinouts.ru/NetworkCables/ethernet_10_100_1000_pinout.shtml):

| Name | Speed | Standard | Wires used | Comments |
| --- | --- | --- | --- | --- |
| 10BASE-T | 10Mb/s | 802.3i | 2 pairs: pins 1,2,3,6 | Runs over four wires on a Category 3 or Category 5 cable. |
| 100BASE-TX | 100Mb/s | 802.3u | 2 pairs: pins 1,2,3,6 | CAT5 copper cabling with two twisted pairs. |
| 1000BASE-T | 1000Mb/s | 802.3ab | 4 pairs: pins 1,2,3,4,5,6,7,8 | At least Category 5 cable, with Category 5e strongly recommended copper cabling with four twisted pairs. Each pair is used in both directions simultaneously |
| 10GBASE-T | 10000Mb/s | 802.3an | 4 pairs: pins 1,2,3,4,5,6,7,8 | Uses category 6a cable. |

## Ethernet UTP cable length:

Each network segment cable length may be up to 100 meters, although several chip manufacturers claim 150 meters. Autonegotiation is a requirement for using 1000BASE-T, according to the standard. Several device drivers will allow you to force 1000 Mbps full duplex to eliminate autonegotiation issues.

## Some UTP network cabling history

Category 5 cable , commonly known as Cat 5, is an unshielded twisted pair cable type designed for high signal integrity. With the 2001 introduction of the TIA/EIA-568-B standard, the category 5 cabling specification was obsoleted and superseded by the category 5e specification.

The original specification for category 5 cable was defined in ANSI/TIA/EIA-568-A, with clarification in TSB-95. These documents specified performance characteristics and test requirements for frequencies of up to 100 MHz. Category 5 cable included four twisted pairs in a single cable jacket. It was most commonly used for 100Mbit/s networks, such as 100BASE-TX Ethernet, although IEEE 802.3ab defined standards for 1000BASE-T - gigabit Ethernet over category 5 cable. Cat 5 cable typically had three twists per inch of each twisted pair of 24 gauge copper wires within the cable. The twisting of the cable reduces electrical interference and crosstalk.

Cat 5e cable is an enhanced version of Cat 5 that adds specifications for far-end crosstalk, usually used for gigabit ethernet. Cat 5e cable does not enable longer cable distances for Ethernet networks: horizontal cables are still limited to a maximum of 90 m in length. Cat 5e cable performance characteristics and test methods are defined in TIA/EIA-568-B.2-2001.

The enhanced version Category 6a cable is defined at frequencies up to 500 MHz and needed to reach 10Gb/s with 10GBASE-T Ethernet.